# 📋 10-Point Annual PM Checklist & Maintenance Log

### 1. WATER HEATER AGE

- [ ] Locate the manufacturer's sticker.

- [ ] Verify installation date is under 10 years old.

- [ ] Plan for replacement if it’s approaching its double-digit birthday.

- [ ] Check the floor pan for rust or moisture.

### 2. TEST SHUT-OFF VALVES

- [ ] "Exercise" (turn off/on) all valves under sinks and behind toilets.

- [ ] Ensure the main water shut-off handle moves freely.

- [ ] Inspect for green corrosion or slow drips.

### 3. REPLACE SUPPLY HOSES

- [ ] Confirm all sink, toilet, and laundry hoses are braided stainless steel.

- [ ] Check for kinks or frayed metal braiding.

- [ ] Replacement Rule: Swap out every 5 years regardless of look.

### 4. CHECK AC DRAIN & FLOAT

- [ ] Clear the condensate line (flush with vinegar or use a shop vac).

- [ ] Manually lift the float switch to ensure the AC unit shuts off.

- [ ] Clean the area around the air handler.

### 5. INSPECT GROUT & CAULKING

- [ ] Check for cracks in the shower floor and wall grout.

- [ ] Inspect the caulk seal around the base of the tub and toilet.

- [ ] Look for "pinholes" where water could seep behind tiles.

### 6. CLEAN BALCONY DRAINS

- [ ] Clear all leaves, dirt, and debris from the drain opening.

- [ ] Pour a gallon of water down the drain to verify it flows away quickly.

- [ ] Check sliding door tracks for blockage.

### 7. TEST LEAK DETECTORS

- [ ] Replace batteries in all Wi-Fi or local alarms.

- [ ] Trigger a "test" to ensure notifications reach your phone.

- [ ] Ensure sensors are placed near the water heater and AC unit.

### 8. INSPECT APPLIANCES

- [ ] Dishwasher: Check the kickplate for signs of slow leaks.

- [ ] Washing Machine: Look for bulging rubber hoses or weeping seals.

- [ ] Refrigerator: Vacuum the coils on the back/bottom for efficiency.

### 9. CHECK ELECTRICAL PANEL

- [ ] Look for charred marks or signs of overheating.

- [ ] Ensure all breakers are labeled clearly.

- [ ] Flip breakers off and on to prevent "sticking" over time.

### 10. SCHEDULE PROFESSIONAL INSPECTIONS

- [ ] Book an annual HVAC "Clean and Check."

- [ ] Arrange a professional plumbing walk-through to catch hidden leaks.

## 📅 Maintenance Completion Log

| Item | Item | Date Completed | Status / Notes | Next Due Date |
| --- | --- | --- | --- | --- |
| 1 | WATER HEATER AGE |  |  |  |
| 2 | TEST SHUT-OFF VALVES |  |  |  |
| 3 | CHECK SUPPLY HOSES |  |  |  |
| 4 | CHECK AC DRAIN & FLOAT |  |  |  |
| 5 | INSPECT GROUT & CAULKING |  |  |  |
| 6 | CLEAN BALCONY DRAINS |  |  |  |
| 7 | TEST LEAK DETECTORS |  |  |  |
| 8 | INSPECT APPLIANCES |  |  |  |
| 9 | CHECK ELECTRICAL PANEL |  |  |  |
| 10 | SCHEDULE PROFESSIONAL INSPECTIONS |  |  |  |